CLF-CAM-01-SV-CNG-Siemens NX

WP6 COLLABORATIVE LEARNING FACTORY

Funded by the European Union. Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or the European Education and Culture Executive Agency (EACEA). Neither the European Union nor EACEA can be held responsible for them.

This work is licensed by the LCAMP Partnership under a Creative Commons Attribution-NonCommercial 4.0 International License.

LCAMP partners:

TKNIKA – Basque VET Applied Research Centre, CIFP Miguel Altuna LHII, DHBW Heilbronn – Duale Hochschule, Baden-Württemberg, Curt Nicolin High School, AFM – Spanish Association of Machine Tool Industries, EARLALL – European Association of Regional & Local Authorities for Lifelong Learning, FORCAM, CMQE: Association campus des métiers et des qualifications industrie du future, MV: Mecanic Vallée, KIC: Knowledge Innovation Centre, MADE Competence Centre Industria 4.0;  AFIL: Associazione Fabbrica Intelligente Lombardia, SIMUMATIK AB; Association HVC Association of Slovene Higher Vocational Colleges; TSCMB:Tehniški šolski center Maribor, KPDoNE: Kocaeli Directorate Of National Education; GEBKİM OIZ  and CAMOSUN college.

CONTENT TABLE

1.	Introduktion	4

2.	Komma igång.	5

3.	Kontrollera delarna	6

4.	Lägg till råämne och Part	7

5.	Placera ditt råämne och Part	8

6.	Placera ut koordinatsystemet	10

7.	Definiera vad som är part och ämne	12

8.	Gå in i 2D	13

9.	Sätt begränsningar	14

10.	Bildregister	15

Introduction

This micro-course covers how to get started with CAM in Siemens NX and how to create a setup for turning. The course is designed to provide a basic understanding of the tools and techniques needed to use CAM in Siemens NX effectively. By following this course, you will learn how to create a setup for turning, which is an important part of the manufacturing process. The course is suitable for both beginners and those looking to refresh their knowledge in the field.

getting started

Start by opening the Manufacturing mode with reference to the part you want to create a CAM for.

Open a part in NX that you want to create a CAM for, then select File > New > Manufacturing. Next, choose the machine you wish to use for your CAM setup.

At the bottom of the window, you can see if the setup is referencing a part—this should be the part you intend to generate a CAM for.

If the correct part is not selected, or if there is no part in the box, click the small icon to the right of the box and choose the part you want to use.

Click OK

Figure 1 Create a file

control the parts

Check if there is already a part and stock added to the setup.

Red arrow = Assembly Navigator

Blue arrow = Testpart

Green arrow = stock material

Figure 2 Control the parts

Open the Assembly Navigator  check whether a part and a stock material have already been added to the setup.

Add stock material and  Part

If the setup already contains a stock and a part, replace them as follows:

Right-click on the test part that is already added  click Replace Component  Select the Part you wish to create a CAM for (your reference part)  Click OK.

Right-click on the stock that is already added  Click Replace Component  Select the stock you have created for your part  Click OK.

Click the small plus sign to the left of Constraints to display all existing constraints  If there are any existing constraints, select them and delete them.

If the setup does not already contain any stock or part

Go to Geometry and select Add Component to add your part and your stock material.

Figure 3 Constraints

Place your stock material and part

To position your stock material and part correctly in the machine:

Red arrow = Geometry

Green arrow = Assembly constraints

Orange arrow = Touch align

Black arrow = Center axis

Click on Geometry  Select Assembly Constraints  choose the constraint type touch align first select the center axis of the workpiece, then the center axis of the chuck.

Figure 4 Assembly Constraints

Figure 5 Types of Constraints

When you have clicked on both center axes, press Apply  select Touch Align again  First, select the bottom/short side of the stock, then select the jaws  press OK.

Orange surface = Underside of stock

Red surface = Surface of the jaws to mark

Red ring = The Jaws

Figure 6 Constraints to attach the stock to the jaws

Once the constraints for the stock are set, it's time to apply the constraints to the part.

Start by marking the center axes of the part in the same way as you did with the stock, using Touch Align (remember to select the part’s centerline first since the first item selected will be the one that moves)  once both are marked, press Apply  choose the Distance constraint type  first select the top of the part, then the top of the stock  set the distance so the part sits 1 mm inside the stock  press OK

Black arrow = Distance

Green arrow = the top of the part

Red sruface = the top of the stock

Blue arrow = where you set the distance

Figure 7 Constraints between part and stock

set the coordinate system

Red arrow = Operation Navigator

Orange arrow = Geometry View

Black arrow = MCS Main Spindle

Click on Operation Navigator  choose Geometry View  choose MCS Main Spindle

Figure 8 Navigate the coordinate system

Red arrow = Main

Green arrow = Dynamic

Black arrow = ZM-Axis

After you opened MCS Main Spindle  make sure the window down the right Specify MCS is chosen as Dynamic  grab the ZM – Axis and drag it to the surface of your part  Click OK

Figure 9 Move the coordinate system

Define what’s the part and the stock

Now we need to tell NX what our Part is and what is our Raw Material/ stock.

Red arrow = Part choosen

Green arrow = Part not choosen

Once the part is selected, you can click on the flashlight and the selected part will be highlighted in the assembly.

Specify Part/ Black arrow = Your part (easiest to select by hovering the mouse over and selecting from the list)

Specify Blank/ Orange arrow = Your stock

Specify Check/ Blue arrow = The Jaws (make sure to choose all the jaws)

Double click on Workpiece Main  Click on the windows and to define.

Figure 10 Define part and stock

Press OK

Entering 2D

You are now in Workpiece Main  Open Turning Workpiece Main by doubleclicking  Press OK

Go back to Assembly Navigator  uncheck the small windows to the left of the stock and the part which should make these visible in 2D with a clip section view.

Red arrow = Un checked part

Orange arrow = Un checked stock

Figure 11 Unview the part and the stock

set limitations

Red arrow = Avoidance External

Black arrow = The tab From Point (FR)

Go back to Operation Navigator  choose Geometry View again  click on Avoidance external  go to the tab From Point (FR)  Move and set the placements for the points that comes up with the part and the stock.

Figure 12 Set limitations

Arrow 1 = From Point, placed a little diagonally above the right edge of the subject

Arrow 2 = Start Point, placed a little bit diagonally above the right edge of the subject

Arrow 3 = Radial clear, is placed a little above the workpiece and a little to the right of the chuck

Arrow 4 = Axial Clear, placed along the center axis and a little to the right of the right edge of the blank

3

2

4

11

Figure 13 set the dots for limitations

Now you are ready to start creating Operations.

Index of images

Figure 1 Create a file	5

Figure 2 Control the parts………………………………….……………………………………………………...6

Figure 3 Constraints……..…………………………………………………………………………………………7

Figure 4 Assembly Constraints………………….………………………………………………………………..8

Figure 5 Types of Constraints………….………………………………………………………………………….8

Figure 6 Constraints to attach the stock to the jaws………………..……………………………………………9

Figure 7 Constraints between part and stock……………………….…………………………………………...9

Figure 8 Navigate the coordinate system……………………………………………………………………….10

Figure 9 Move the coordinate system…………………………………………………………………………...11

Figure 10 Define part and stock….………………...……………………………………………………………12

Figure 11 Unview the part and the stock………………………………………………………..………………13

Figure 12 Set limitations…………………………………………………………………………..…….………..14

Figure 13 Set the dots for limitations…………………………………………………………………..………..14