Spearfish

Code: ELEQ0126V

Professional Family:

Electrical & Electronics

Professional Area:

Electronic Equipment

🔧 Guided Activity: Marlin Firmware Modification and BLTouch Integration into Ender 3

🕒 Total duration: 4 hours

👥 Class organization:

- 4 initial groups → are combined into 
2 large groups (each with 2 printers).

- Each large group is divided into 2 subgroups:

  - Subgroup A: Modifies the Marlin firmware and integrates the BLTouch.

  - Subgroup B: Validate behavior using Pronterface.

✅ STEPS TO FOLLOW (guided by the teacher)

🔹 STEP 1 – Prepare the work environment (40 minutes)

1.1 Install Arduino IDE.

1.2 Install Visual Studio Code (VSC).

1.3 In VSC, install the PlatformIO extension.

1.4 In VSC, install the Auto Build Marlin extension.

📝 The teacher guides the installation step by step and makes sure that all students have the tools ready.

🔹 STEP 2 – Download and configure the Marlin firmware (50 minutes)

2.1 Download the latest version of Marlin from its official website (or from the repository indicated).

2.2 Download the sample configuration package for the Ender 3 (per exact model).

2.3 Copy the Configuration.h and Configuration_adv.h files  from the example to the Marlin/ folder  of the downloaded project.

2.4 Open the root folder of the project in PlatformIO (VSC).

🛠️ Subgroup A performs this task while subgroup B prepares the commands to test the printer with Pronterface (see step 5).

🔹 STEP 3 – Modify the firmware to integrate BLTouch (45 minutes)

3.1 Activar el sensor BLTouch en Configuration.h:

#define BLTOUCH

3.2 Verify that Bilinear Auto Bed Leveling is enabled:

#define AUTO_BED_LEVELING_BILINEAR

3.3 Enable Z Safe Homing:

#define Z_SAFE_HOMING

3.4 Configure the  BLTouch offsets (approximate values, then adjusted):

#define NOZZLE_TO_PROBE_OFFSET { X, Y, Z }

3.5 Set Mesh Point Number, Bed Size, Base Plate Type, etc.

📌 The teacher can show a basic example on the screen to guide everyone.

🔹 STEP 4 – Compile and generate the firmware file (30 minutes)

4.1 In Auto Build Marlin, select the correct build environment (e.g. STM32F103RET6_creality).

4.2 Compiling the firmware.

4.3 If the build is successful, generate the .bin or .hex file.

4.4 Copy the generated file to the microSD card.

4.5 Insert the SD into the printer and update the firmware.

✅ 
Verify that the printer turns on correctly, without errors on the screen.

🔹 STEP 5 – Validation with Pronterface (30 minutes)

📌 Now subgroup B intervenes using Pronterface and communicating via USB with the modified printer.

5.1 Send commands to check that the printer responds correctly:

- M503: View the current configuration values.

- G28: Homering all the axes.

- G29: Begin self-leveling (must lower and raise the BLTouch).

- M851: View and adjust Z-offset.

- M500: Save the values.

5.2 Compare the behavior with the unmodified printer 
       (control), observing:

- Leveling accuracy.

- Behavior when homeing the Z axis.

- Quality of movement.

🔹 STEP 6 – Error Analysis and Troubleshooting (35 minutes)

6.1 If the firmware is not working properly:

- Review the pin settings.

- Check if the BLTouch is properly connected.

- Readjust offsets or mesh parameters.

- Recompile and reload the firmware.

6.2 If there are errors in Pronterface:

- Analyze with M122, M119, etc.

- Consult group A for possible adjustments.

📌 Each subgroup takes note of what has gone wrong and what solution it has applied.

🔹 STEP 7 – Final Comparison and Review (30 minutes)

7.1 Compare modified printer with reference printer.

7.2 Assess print quality, self-leveling reliability, and overall performance.

7.3 Document on a sheet or quick presentation:

- Changes applied

- Problems detected

- Solutions provided

- Upshot

🗣️ Each group presents its experience in 5 minutes.