WP6 COLLABORATIVE LEARNING FACTORY

3D PRINTERS

Funded by the European Union. Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or the European Education and Culture Executive Agency (EACEA). Neither the European Union nor EACEA can be held responsible for them.

This work is licensed by the LCAMP Partnership under a Creative Commons Attribution-NonCommercial 4.0 International License.

LCAMP partners:

TKNIKA – Basque VET Applied Research Centre, CIFP Miguel Altuna LHII, DHBW Heilbronn – Duale Hochschule, Baden-Württemberg, Curt Nicolin High School, AFM – Spanish Association of Machine Tool Industries, EARLALL – European Association of Regional & Local Authorities for Lifelong Learning, FORCAM, CMQE: Association campus des métiers et des qualifications industrie du future, MV: Mecanic Vallée, KIC: Knowledge Innovation Centre, MADE Competence Centre Industria 4.0;  AFIL: Associazione Fabbrica Intelligente Lombardia, SIMUMATIK AB; Association HVC Association of Slovene Higher Vocational Colleges; TSCMB:Tehniški šolski center Maribor, KPDoNE: Kocaeli Directorate Of National Education; GEBKİM OIZ  and CAMOSUN college.

Document summary

| Document type: | Public Course |
| --- | --- |
| Title | 3D Printers |
| Author/s | Daniel Guiraudie |
| Reviewer | Camille Léonard |
| Date | Jully 2025 |
| Document status | 1.0 |
| Document level | Confidential until its publication |
| Document description | This course aims to cover the basics and history of additive manufacturing, explore its applications, present some of its principles, and outline a process for printing a part using the fused filament fabrication (FFF) method. |
| Cite this deliverable as: | Guiraudie D., 2025, 3D Printers |
| Document level | Public |

CONTENT TABLE

Executive Summary	5

1.	Introduction	6

1.1.	General Information	6

1.2.	Examples of Parts	6

1.3.	History	8

2.	3D Printing – Applications	9

2.1.	Advantages	9

2.2.	Disadvantages	9

3.	General Approach	10

3.1.	Main Steps	11

3.1.1.	CAD Model Recuperation	11

3.1.2.	Converting the CAD Model	11

3.1.3.	Preparation for Printing	12

3.1.4.	Application	14

4.	Index of Images	17

5.	Index of tables	17

6.	Annexes	18

6.1.	Typology of the Main Principles	18

6.1.1.	Pastic and Composite Materials	18

6.1.2.	Metalic Materials	19

# Executive Summary

In this course (which is not intended to be exhaustive), we will cover the basics and history of additive manufacturing, explore its applications, present some of its principles, and outline a process for printing a part using the fused filament fabrication (FFF) method.

.

# Introduction

## General Information

3D printing offers :

greater design freedom

material savings

localized production

object customization

Indeed, in most cases, traditional manufacturing starts from a raw block from which material is removed to obtain the desired part, whereas additive manufacturing builds the object layer by layer.

The two manufacturing processes are complementary, but the way parts are designed is different. The fields of application are very diverse: aerospace, space industry, automotive, medical, civil engineering…

## Examples of Parts

| Civil Engineering | Automotive | Aerospace |
| --- | --- | --- |
|  | Brake caliper |  |
|  | Interior cockpit prototype | Turbine blade |
| 3D-printed steel bridge |  |  |

- It is possible to customize the product, meet short lead times, produce small series, and create complex shapes—all while reducing costs.

- Evolution graphs

Figure 1. Evolution graphs.

- Global market

Figure 2. Global market

## History

Figure 3. History

Legend translation:

1980: 1st patent application for RP (Rapid Prototyping) technology filed by Dr. Kodama in Japan in May 1980

1986: SLA 1st patent granted to Charles Hull for a stereolithography device

1989: SLS (Select Laser Sintering) Carl Deckard patents selective laser sintering

1990: DMLS (Direct Metal Laser Sintering) EOS develops direct metal laser sintering

1991: LOM (Laminated Object Manufacturing): SOLIDSCAPE develops laminated object manufacturing

MJM (Multijet Modeling): OBJET develops multi-jet modeling

1992: FSM (Fused Deposition Modeling) STRATASYS develops hot-wire deposition

2000: SLM (Selective Laser Melting): MCT Tech develops selective laser melting

2016: MJF (Multi Jet Fusion) HP develops selective heat fusion

# 3D Printing – Applications

## Advantages

Ability to produce complex shapes: Enables the manufacturing of 3D parts without shape being a limiting factor—such as cavities, lattices (lattice structures), thin walls, etc.

Clear weight savings thanks to lattice structures or topological optimization (using material only where it is needed). This also allows for a reduction in the number of final parts by combining several components into a single printed piece, thus reducing subsequent assembly operations.

Mechanical properties under static loads are often equivalent or even superior to those of traditional processes (casting, rolling).

Customization without additional cost: Production of unique, personalized parts—different from one another—on the same build plate and within the same production cycle.

This is particularly valuable in the medical field, for example in creating prosthetics tailored to each patient.

Possibility to functionalize surfaces, optimize fluid management, or shorten production lead times.

## Disadvantages

The cost of production remains the most limiting factor, mainly due to the high cost of machines and their low productivity. The lack of material standards specific to certain sectors also slows the adoption of these processes.

As a result, 3D printing is more suitable for small to medium production runs. Some processes do not allow for certain surface orientations without the addition of supports, which can lead to expensive post-processing.

Vérification

intérêt économique

# General Approach

3D printing encompasses technologies that manufacture parts by successively adding layers of material based on a digital model.

Table 1. General approach

| Basic principle | Detailed principle |
| --- | --- |
| Legend translation: Mesh (STL format) Slicing process 3D CAD model Layer fabrication | Legend translation : Digital data: 3D CAD file - STL File preparation: File correction, orientation, part placement, support, slicing Manufacturing Finishing: Cleaning, support removal, sandblasting, machining,... |

## Main Steps

Digital channel

3D modeling

Post-treatment

3D printing

Route preparation/generation

Conversion

3D printers

Integrated software

Converter

3D modeler

Accessories

Cleaning, sanding...

Surface treatment

Modele

3D CAD: compliance with process-related design rules

Scanning of physical objects

(3D scan)

-3D formats:

STL, OBJ, STEP

-Visualization, analysis and repair of defects

Model orientation

Support generation

Slicing

Path generation (filling and contours)

Machine management (direct, network...)

Material management according to machine autonomy

(Liquids, powders, solids)

-Substrate removal

Figure 3. CAD model recuparation

CAD Model Recuperation

The care and attention taken in preparing the 3D CAD file has a direct impact on the quality of the 3D print.

Ensure that wall thicknesses are compatible with the printing technology.

Laying plane: minimum thickness 0.3 mm

Vertical plane: minimum thickness 0.5 mm

### Converting the CAD Model

It is necessary to convert the model into a format compatible with 3D printers (in this case, only the STL format).

This format is based on facetting using triangular elements (MAILLAGE).

Convert file to stl with fine parameters

Conversion du fichier en stl avec des paramètres larges

Convert file to stl with wide parameters

CAD

Model

Figure 4. Converting the CAD model

You need to reduce the risk of having surfaces with facets that are too visible, while limiting the size of the STL file, which has an impact on how quickly it can be managed by the preparation software.

Correct the mesh if necessary (remove any surface intersections).

Figure 5. Converting the CAD model, 2nd image

Preparation for Printing

Importance of part orientation on the machine table

Table 2. Preparation for printing

| Printing time is generally proportional to part height on the Z axis | Limiting the amount of media | Improve surface quality and mechanical properties |
| --- | --- | --- |
| Slower Faster | Need support No support | Fragility between layers Better resistance |

Slicing is the transformation of the model into layers of constant or variable thickness. It influences manufacturing time and part precision.

Fewer, thicker layers = faster, but more fragile

Fewer, thinner layers = slower, but more resistant

The support is often made of cellular or hollow structures, and is used to support the overhanging parts of a part being manufactured.

Figure 6. Preparation for printing 2nd image

Figure 7. Preparation for printing 1st image

Path generation takes into account layer geometry and determines process parameters (nozzle diameter, flux, laser power, speed, etc.).

Table 3. Peparation for printing 2nd table

| The contours of the model influence the resistance | Inside the model: adjusting the filling intensity | Example |
| --- | --- | --- |
| The outline is made with 3 layers | Full Spaced Spaced One-way Two-way |  |

### Application

Software used: CAD    Solidworks        3D printer Ultimaker

Figure 8. Application, front part

Part : Front part

#### STL format conversion

Figure 9. STL format conversion

#### Orientation on 3D machine plate

Use symbols

Print time   +++    ++     +

Support: +++      ++      +

Table 4. Orientation on 3D machine plate

| Print time: support | Print time: support | Print time: support |
| --- | --- | --- |

#### Generation path

Parameters related to the generation of paths:

- Layer thickness

- Wall thickness (wall, shell, etc.)

- To the filling

- Print speed

- Support type

- Machine nozzle

Here 0.4 mm

Figure 10. Generation path

After slicing the part, visualization of the layers and supports

Figure 11. Generation path

#### Post-processing

Remove all supports

Check the appearance of the surfaces

Check the dimensions

Verify the geometry

# Index of Images

Figure 1. Evolution graphs.	7

Figure 2. Global market	8

Figure 3. CAD model recuparation	11

Figure 4. Converting the CAD model	12

Figure 5. Converting the CAD model, 2nd image	12

Figure 6. Preparation for printing 2nd image	13

Figure 7. Preparation for printing 1st image	13

Figure 8. Application, front part	14

Figure 9. STL format conversion	14

Figure 10. Generation path	16

Figure 11. Generation path	16

Figure 12. Molten wire printing	18

Figure 13. Resin Polymerization Printing	19

Figure 14. Impression par 3dp binder jetting	19

Figure 15. Laser melting on a powder bed	20

Figure 16. Laser Fusion on Powder 2nd image	21

Figure 17. Laser Fusion on Powder 1st image	21

# Index of tables

Table 1. General approach	10

Table 2. Preparation for printing	12

Table 3. Peparation for printing 2nd table	14

Table 4. Orientation on 3D machine plate	15

# Annexes

## Typology of the Main Principles

### Pastic and Composite Materials

      - Molten wire printing

A thermoplastic-based filament is heated and extruded through a nozzle to form successive layers creating a part

filament

nozzle

Figure 12. Molten wire printing

#### Resin Polymerization Printing

A beam of UV rays is projected by a laser to polymerize a photosensitive resin contained in the 3D printer's tray, forming a solid layer.

With each layer, the tray is lowered and printing continues.

Figure 13. Resin Polymerization Printing

#### Impression par 3dp binder jetting

A print head deposits a binder on a layer of powder in a selective process that is repeated until a complete part is formed. The unbound powder can be used as a support.

With each layer, the platen is lowered and printing continues.

Once the part is complete, the unbonded powder is removed to reveal the finished object.

Figure 14. Impression par 3dp binder jetting

### Metalic Materials

#### Laser melting on a powder bed

The powder (5 to 45µm) is spread by a roller or scraper, in layers of 20 to 100µm, on a preheated plate (200°C).

The laser beam (from 50 to 1000W, 400W standard) scans (1m/s) the section of the part to be manufactured.

Figure 15. Laser melting on a powder bed

Legend translation:

Enceinte de fabrication sous gaz: Gas manufacturing enclosure

Laser: Laser

Miroirs d’orientation xy : Xy orientation mirrors

Flux de gaz: Gas flow

Pièce en fabrication = la pièce et ses supports sont soudés au plateau de fabrication : Part being manufactured = the part and its supports are welded to the manufacturing platform

Racloir d’étalement de la poudre : Powder spreading scraper

Réservoir de poudre : Powder tank

Plateau de fabrication: Manufacturing platform

#### 4.1.2.2 Laser Fusion on Powder

The laser-melted powder is deposited on a plate to manufacture a complex part

On a part to add a function or repair

Deposition bead width from 1 to 5 mm height from 0.3 to 2mm

Figure 16. Laser Fusion on Powder 2nd image

Figure 17. Laser Fusion on Powder 1st image

.