WP6 COLLABORATIVE LEARNING FACTORY

DRAWING UP A MANUFACTURING RANGE

Funded by the European Union. Views and opinions expressed are however those of the author(s) only and do not necessarily reflect those of the European Union or the European Education and Culture Executive Agency (EACEA). Neither the European Union nor EACEA can be held responsible for them.

This work is licensed by the LCAMP Partnership under a Creative Commons Attribution-NonCommercial 4.0 International License.

LCAMP partners:

TKNIKA – Basque VET Applied Research Centre, CIFP Miguel Altuna LHII, DHBW Heilbronn – Duale Hochschule, Baden-Württemberg, Curt Nicolin High School, AFM – Spanish Association of Machine Tool Industries, EARLALL – European Association of Regional & Local Authorities for Lifelong Learning, FORCAM, CMQE: Association campus des métiers et des qualifications industrie du future, MV: Mecanic Vallée, KIC: Knowledge Innovation Centre, MADE Competence Centre Industria 4.0;  AFIL: Associazione Fabbrica Intelligente Lombardia, SIMUMATIK AB; Association HVC Association of Slovene Higher Vocational Colleges; TSCMB:Tehniški šolski center Maribor, KPDoNE: Kocaeli Directorate Of National Education; GEBKİM OIZ  and CAMOSUN college.

Document summary

| Document type: | Public Course |
| --- | --- |
| Title | Drawing up a Manufacturing Range |
| Author/s | Christophe Belmon |
| Reviewer | Camille Léonard |
| Date | Jully 2025 |
| Document status | 1.0 |
| Document level | Confidential until its publication |
| Document description |  |
| Cite this deliverable as: | Belmon C., 2025, Drawing up a Manufacturing Range |
| Document level | Public |

CONTENT TABLE

Executive Summary	7

1.	course OBJECTIve	8

2.	The process	8

3.	Course	9

3.1.	Definitions	9

3.2.	Morphological Study of the Machined Part	10

3.2.1.	The first machined surface must provide maximum stability for the subsequent machining operations.	10

3.2.2.	The part resting on the first machined surface, must be able to be effectively clamped.	10

3.3.	Analysis of Critical Specifications	10

3.3.1.	Intrinsic dimensions for which: IT < 0,05 mm	10

3.3.2.	Dimensions between surfaces for which:  IT < 0,1 mm	10

3.3.3.	Geometric tolerances for which: IT ~ 0,1 %.	10

3.3.4.	Surface roughness: Ra ≤ 1,6.	10

3.4.	Decisions for Making Intrinsic Surfaces	10

3.4.1.	Role of the Operations	11

3.4.2.	The dimensional and geometric quality of the surface depends on the consistency of the finishing chip thickness, as well as the rigidity of the tools, machines, and parts	11

3.4.3.	Frontier between cutting tool and abrasive tool	12

3.4.4.	Generating processes should be preferred over form processes for surfaces with tight intrinsic specifications.	12

3.4.5.	To achieve the desired surface roughness, the feed rate must be selected according to the tool nose radius.	12

3.5.	Decisions on the Grouping of Surface Machining	12

3.5.1.	For two surfaces A and B linked by tight specifications, the following should be applied:	12

3.5.2.	All accessible surfaces must be machined in the same phase	12

3.5.3.	Finishing tools, which are fragile or expensive, must begin cutting, or even exit, On pre-machined (prepared) surfaces	13

3.6.	Order of Operations	13

3.6.1.	Finishing tools, being fragile or expensive, should operate for the shortest possible time, therefore over a minimal cutting path.	13

3.6.2.	Tool entry and exit should be facilitated. To do this, plan for:	13

3.6.3.	Machining operations that weaken the part should be postponed as late as possible.	14

3.6.4.	The cutting edges of deformable tools (such as grooving tools or drills) must be loaded symmetrically.	14

3.6.5.	In the case of intersection of surfaces the burr must be rejected:	14

3.6.6.	Unless another rule contradicts it, the operation order should aim to allow the use of the most rigid tools possible.	14

3.7.	Order of Phases	15

3.7.1.	In phase 10, the surface or group of surfaces must be machined to ensure the most stable position possible and good clamping in subsequent phases.	15

3.7.2.	If reworking is possible and equivalent in terms of stability on a plane or on a long cylinder, the support on the plane should be chosen.	15

3.7.3.	The first three phases must enable the necessary and sufficient rework surfaces to be machined for subsequent positioning.	15

3.7.4.	Beyond phase 20 or 30, a recovery on raw should only be exceptional	15

3.7.5.	In the case where two surfaces A and B are positioned relative to each other (geometry, position, clamped side) and can be machined in relation to one another, the surface that offers the best rework stability and the possibility of correct clamping must be machined first.	15

3.7.6.	Machining of fragile and precise surfaces—threads, ground surfaces, etc. should be delayed until later in the manufacturing process.	15

3.7.7.	A precise finish must be removed from a blank which has given off too much heat.	15

3.8.	Selection of Starting Surfaces	16

3.8.1.	During the initial phases, the choice of positioning must allow for optimal distribution of material around the machined areas.	16

3.8.2.	The positioning must be chosen to provide the best stability of the part during cutting.	16

3.8.3.	Positioning dispersions should be kept to a minimum by positioning on good quality rough surfaces	16

3.8.4.	The supports must oppose the cutting forces	16

3.9.	positioning	16

3.9.1.	La Positioning must be isostatic.	16

3.9.2.	The support points defining a surface or a basic line must be as far apart as possible.	16

3.9.3.	Rule 1.8.4 remains strictly applicable	16

3.9.4.	The relative position of the elementary surfaces involved in positioning must allow the use of standard tooling.	16

3.9.5.	Positioning and clamping elements must never interfere with the tool path.	16

3.10.	Clamping	16

3.10.1.	The position of the clamping points and the intensity of clamping must be such that the workpiece remains stable under the cut	17

3.10.2.	Under no circumstances should the actions of the clamping on the part cause deformations that would result in the rejection of the part	17

3.11.	Additional rules concerning machining on MOCN	17

3.11.1.	When a tool is in place on its workstation, it must perform as many operations as possible	17

3.11.2.	Operations must be organised in such a way as to reduce tool cycle times.	17

3.11.3.	Drilling and reaming operations are carried out using machining cycles.	17

3.11.4.	Contouring, it is possible to achieve a quality grade of 6–7 (finishing pass).	17

3.11.5.	The part will be produced in a minimum number of phases.	17

3.11.6.	On a machining center, the objective will be to access as many part faces as possible in the same phase:	17

3.11.7.	If reworking is performed on a CNC machine, positioning deviations will be eliminated by:	18

3.11.8.	Tool cycles should be as short as possible	18

4.	Guided workS	18

4.1.	Analysis of the Part	20

4.1.1.	Analyse the morphology of the part	20

4.1.2.	To analyse drawing spécifications	20

4.2.	Define surface groupings	21

4.2.1.	Identify machining directions	21

4.2.2.	Complete the surface groupings table below	21

4.2.3.	Make decisions on the production of intrinsic surfaces	21

4.3.	Define Operation Groupings	22

4.4.	Order the Machining Phases	22

4.4.1.	Propose an Preliminary Manufacturing Study	22

4.4.2.	PROPOSE ANOTHER Preliminary Manufacturing Study	23

5.	References	24

6.	Index of Images	25

7.	Index of tables	25

8.	Annexes	26

8.1.	Annex 1	26

8.2.	Annex 2: CORRECTION	30

8.2.1.	Analysis of the part	30

8.2.2.	Defining Surfaces Groupings	31

8.2.3.	Define Operation Groupings	32

8.2.4.	Order the Machining Phases	33

Draw up the machining range (Fill in a range sheet for each Preliminary Manufacturing Design)	33

(ABSTRACT) Lorem ipsum dolor sit amet, consectetuer adipiscing elit, sed diam nonummy nibh euismod tincidunt ut laoreet dolore magna aliquam erat volutpat. Ut wisi enim ad minim veniam, quis nostrud exerci tation ullamLorem ipsum dolor sit amet, consectetuer adipiscing elit, sed diam nonummy nibh euismod tincidunt ut laoreet dolore magna aliquam erat volutpat. Ut wisi enim ad minim veniam, quis nostrud exerci tation ullam.

.

Executive Summary

# course OBJECTIve

The general rules outlined below will guide you as reliably as possible from the product definition drawing to the drafting of preliminary manufacturing study projects.

Despite similarities and analogies, each part must be considered a unique case.

Every manufacturing study is therefore dependent on:

the part itself

the production rate

the available equipment

the available personnel

the deadlines

the economic climate

These rules will apply to:

parts that do not undergo heat treatment; after treatment, parts may only be machined using abrasive tools, electrical discharge machining (EDM), or high-speed machining (HSM)

parts whose deformation due to cutting forces is negligible

parts machined on conventional machine tools

parts machined on CNC (computer numerical control) machines (see rules in the annexes)

# The process

The manufacturing analysis involves 9 steps, which, thanks to the application of established rules and a number of decisions and compromises, should lead to the general scheduling of the manufacturing process.

Table 1. Nine steps of the manufacturing analysis

| STEP | OBJECTIVE |
| --- | --- |
| 1) Morphological study of the machined part | Make a likely initial choice of the first phase |
| 2) Analysis of critical specifications | Identify the risks of rejects |
| 3) Decision on the production of intrinsic surfaces | Ensure surface quality |
| 4) Decision on grouping of surface machining | Limiting rework gaps and handling |
| 5) Order of operations | Ensure part quality and minimize cost |
| 6) Order of phases | Ensure part quality and minimize cost |
| 7) Choice of starting surfaces | Ensure the best possible "balancing" of the raw part |
| The following two steps should enable proper drafting of the preliminary phase plans: |  |
| 8) Positioning | Reducing recovery gaps |
| 9) Clamping | Ensure the stability of the part during cutting |

# Course

## Definitions

Workstation: Machine equipped to carry out a set of operations.

Operation: Execution of one or more basic surfaces, performed without dismounting the part.

Phase: A set of operations (possibly grouped into sub-phases) carried out at the same workstation. They are usually referred to by the name of the machine on which the operations are performed. Example: Example: Phase 10 TURNING.

Sub-phase: A set of operations performed without dismounting the part.

Figure 1. Sub-phase A: Chuck: hard jaws.

Figure 2. Sub-phase B: chuck: 3 soft jaws

Sub-phase A: chuck: 3 hard jaws		                  Sub-phase B: chuck: 3 soft jaws

## Morphological Study of the Machined Part

OBJECTIVE: To make a priori a probable choice of the first phase.

### The first machined surface must provide maximum stability for the subsequent machining operations.

Note: A cylinder can be considered dominant over a plane when:

Projected surface area of the cylinder ≥ 1,5 surface area of the plane.

A cylinder can be considered usable for reworking when:

Length of the cylinder ≥ à 1,25 Ø cylinder.

### The part resting on the first machined surface, must be able to be effectively clamped.

## Analysis of Critical Specifications

OBJECTIVE: To identify the risks of scrap.

Only the following specifications will be considered as critical to achieve, and therefore analysed accordingly:

### Intrinsic dimensions for which: IT < 0,05 mm

### Dimensions between surfaces for which: 	IT < 0,1 mm

### Geometric tolerances for which: IT ~ 0,1 %.

### Surface roughness: Ra ≤ 1,6.

## Decisions for Making Intrinsic Surfaces

OBJECTIVE: to ensure the quality of surfaces.

Focus is placed on the surfaces mentioned in sections 3.3.1 and 3.3.4.

The choice of the number of operations and the number of tools will be deduced from this analysis.

### Role of the Operations

Roughing: Removes excess material and prepares the part for subsequent operations.

Semi-finishing: Shapes the surface and defines its position.

Finishing: gives the surface its dimensions and roughness.

EACH OPERATION REQUIRES A DIFFERENT TOOL

### The dimensional and geometric quality of the surface depends on the consistency of the finishing chip thickness, as well as the rigidity of the tools, machines, and parts

The pass preceding the finishing operation must establish the geometry of the surface

Therefore, in general and depending on the condition of the raw part, the decision will be:

  - 3 operations for quality < 8

- 2 operations for quality < 9–10

- 1 operation for quality > 10

Table 2. Table to Assist in Choosing the Number of Machining Operations

| CRITERIA | 1 OPERATION | 2 OPERATIONS | 3 OPERATIONS | 4 OPERATIONS |
| --- | --- | --- | --- | --- |
| IT > 0,4 |  |  |  |  |
| 0,15 < IT < 0,4 |  |  |  |  |
| 0,05 < IT < 0,15 |  |  |  |  |
| IT < 0,05 |  |  |  |  |
| Quality > 12 |  |  |  |  |
| Quality 9-11 |  |  |  |  |
| Quality 7-8 |  |  |  |  |
| Quality < 7 |  |  |  |  |
| Ra < 0,8 | Correction to be expected |  |  |  |

QUALITIES LESS THAN OR EQUAL TO 6 REQUIRE CORRECTION OR EVEN SUPERFINISHING.

### Frontier between cutting tool and abrasive tool

Grinding is intended for heat-treated parts.

Depending on the risk of deformation and the required surface roughness, grinding can be carried out up to grade 10.

For untreated parts, grinding is essential:

For external surfaces with quality grades < 7

For internal surfaces with quality grades < 6 (grade 7 can be achieved with a reamer, if possible)

### Generating processes should be preferred over form processes for surfaces with tight intrinsic specifications.

### To achieve the desired surface roughness, the feed rate must be selected according to the tool nose radius.

## Decisions on the Grouping of Surface Machining

OBJECTIVE: Minimize handling and reworking deviations.

Rule imposed by an analysis of the geometry and relative dimensions of the surfaces.

### For two surfaces A and B linked by tight specifications, the following should be applied:

- Machine A and B in the same phase

If that is not possible:

- Machine B by positioning on A

If that is still not possible:

- Machine both surfaces A and B by positioning on the same reference surface.

Rules imposed by combinations related to machine tools and cutting tools.

OBJECTIVE: To reduce the number of phases and idle times.

### All accessible surfaces must be machined in the same phase

Association principle:

#### Technological criteria

When multiple surfaces are generated simultaneously:

Table 3. Plane cylinder and plan-plan connections

| PLANE CYLINDER CONNECTION | PLAN-PLAN CONNECTION |
| --- | --- |

#### Economic Criteria

The grouping of certain surfaces reduces machining costs:

Figure 3. Grouping of surfaces associated with machining costs

### Finishing tools, which are fragile or expensive, must begin cutting, or even exit, On pre-machined (prepared) surfaces

## Order of Operations

Objective: Ensure part quality at minimum cost.

### Finishing tools, being fragile or expensive, should operate for the shortest possible time, therefore over a minimal cutting path.

### Tool entry and exit should be facilitated. To do this, plan for:

Entry chamfers for finishing tools, taps, and reamers.

Clearance volumes for boring tools, taps, reamers, and grinding wheels

### Machining operations that weaken the part should be postponed as late as possible.

### The cutting edges of deformable tools (such as grooving tools or drills) must be loaded symmetrically.

### In the case of intersection of surfaces the burr must be rejected:

Either on the surface on which it can be tolerated (which imposes the order of finishes).

Or on the surface accessible for deburring.

### Unless another rule contradicts it, the operation order should aim to allow the use of the most rigid tools possible.

## Order of Phases

objective : To ensure part quality at minimum cost.

The rules relating to operations remain applicable to the phases.

### In phase 10, the surface or group of surfaces must be machined to ensure the most stable position possible and good clamping in subsequent phases.

### If reworking is possible and equivalent in terms of stability on a plane or on a long cylinder, the support on the plane should be chosen.

### The first three phases must enable the necessary and sufficient rework surfaces to be machined for subsequent positioning.

### Beyond phase 20 or 30, a recovery on raw should only be exceptional

### In the case where two surfaces A and B are positioned relative to each other (geometry, position, clamped side) and can be machined in relation to one another, the surface that offers the best rework stability and the possibility of correct clamping must be machined first.

### Machining of fragile and precise surfaces—threads, ground surfaces, etc. should be delayed until later in the manufacturing process.

### A precise finish must be removed from a blank which has given off too much heat.

A general outline will be produced before the semi-finishing and finishing phases.

## Selection of Starting Surfaces

objective : Ensure optimal balance.

### During the initial phases, the choice of positioning must allow for optimal distribution of material around the machined areas.

### The positioning must be chosen to provide the best stability of the part during cutting.

### Positioning dispersions should be kept to a minimum by positioning on good quality rough surfaces

### The supports must oppose the cutting forces

## positioning

objectif: Reduce as much as possible the reworking deviations of parts in the workpiece holders.

### La Positioning must be isostatic.

### The support points defining a surface or a basic line must be as far apart as possible.

### Rule 1.8.4 remains strictly applicable

### The relative position of the elementary surfaces involved in positioning must allow the use of standard tooling.

### Positioning and clamping elements must never interfere with the tool path.

## Clamping

objective : To ensure that the complete connection is maintained under the action of the cutting forces.

### The position of the clamping points and the intensity of clamping must be such that the workpiece remains stable under the cut

The fixing point should be placed near the location where cutting forces occur

### Under no circumstances should the actions of the clamping on the part cause deformations that would result in the rejection of the part

En flexion, torsion , flambage.

Au matting.

## Additional rules concerning machining on MOCN

Care must be taken with:

choice of tools (type, material)

cutting conditions (tool life)

swarf removal.

### When a tool is in place on its workstation, it must perform as many operations as possible

### Operations must be organised in such a way as to reduce tool cycle times.

### Drilling and reaming operations are carried out using machining cycles.

### Contouring, it is possible to achieve a quality grade of 6–7 (finishing pass).

### The part will be produced in a minimum number of phases.

### On a machining center, the objective will be to access as many part faces as possible in the same phase:

Through clever part gripping.

By using a rotary table.

To improve workpiece pick-up, the following features are required:

Jaw pick-up or centering on turning parts.

A plane face and possibly two bores for milling parts.

### If reworking is performed on a CNC machine, positioning deviations will be eliminated by:

Probing an already machined surface with a Renishaw-type device integrated into the machine.

A calculation that will allow a change of the program origin.

### Tool cycles should be as short as possible

# Guided workS

The goal of this set of sessions is to design the manufacturing process for the robot's WS4P010 part.

Figure 4. Chassis view

Figure 5. Image for designing the manufacturing process for the robot's WS4P010 part

## Analysis of the Part

### Analyse the morphology of the part

Objective : make the most likely choice for the first phase

Locate the rough and machined surfaces using the marking system defined below

Start with reference surfaces.

Identify surfaces that remain rough and mark them on the Bi drawing.

Identify machined surfaces and mark them on the drawing: plane Pi, cylinder Ci, other Xi.

4

5

1

6

Directions d’usinage

3

2

Figure 6. Analysis of the Part

### To analyse drawing spécifications

OBJECTIVE: identify scrap risks and related surfaces.

#### Complete the analysis table below.

To be completed at 4.2.1 ↓

Table 4. Analysis of drawing specifications

| Reference surface | Reference element | Number | Specification | Tolerance zone | Link with specified reference(s) or other element(s)or other element(s)s) | Machining direction (see 2.1.1) |
| --- | --- | --- | --- | --- | --- | --- |

#### Highlight tricky specifications. That is: IT ≤ 0.05 ; qualité ≤ 7 ; Ra ≤ 0.8 ; zone de tolérance ≤ 0.05

Highlight them in the table on the previous page.

## Define surface groupings

Some surfaces cannot be produced separately; they are grouped by technological requirements (grooving, counterboring, etc.).

### Identify machining directions

Complete the « Machining Directions » column in the table 4.1.2

### Complete the surface groupings table below

### Make decisions on the production of intrinsic surfaces

OBJECTIVE : Ensure surface quality.

Some surfaces may require semi-finishing and finishing operations, or the use of specific machining processes, such as grinding.

Complete the "Comments" column of the surface grouping table.

Table 5. Decisions on the production of intrinsic surfaces

| Machining direction | Surface groupings | Comments |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |

## Define Operation Groupings

OBJECTIVE: limit rework deviations and handling.

Complete the machining operation grouping table.

Table 6. Machining operation grouping table

| Operations grouping | Surfaces |
| --- | --- |

## Order the Machining Phases

Draw up the machining order (fill in one order sheet per Preliminary Manufacturing Study).

### Propose an Preliminary Manufacturing Study

Table 7. Preliminary Manufacturing Study -1  Nomenclature of phases and sub-phases

| Preliminary Manufacturing Study -1 Nomenclature of phases and sub-phases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N° phase | S/ Phase | Designation of phases & operations | Machine | Positioning | Sketch |

### PROPOSE ANOTHER Preliminary Manufacturing Study

Table 8. Preliminary Manufacturing Study -2   Nomenclature of phases and sub-phases phase

| Preliminary Manufacturing Study -2 Nomenclature of phases and sub-phases phase |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N° phase | S/ Phase | Designation of phases & operations | Machine | Positioning | Sketch |

# References

Abele, E., Metternich, J., Tisch, M., Chryssolouris, G., Sihn, W., ElMaraghy, H., . . . Ranz, F. (2015). Learning Factories for Research, Education, and Training. Procedia CIRP, Volume 32, 1-6.

Abele, E., Metternich, J., Tisch, M., Chryssolouris, G., Sihn, W., ElMaraghy, H., . . . Ranz, F. (2015). Learning factories for research, education, and training. Procedia CIRP 32:1–6. doi: https:// doi. org/ 10. 1016/j. procir. 2015. 02. 187

Aegilmez, S. (2021). Forcam Force Edge Manual. (F. gmbh, Producer) Retrieved from https://forcam.com/app/uploads/2021/11/Manual-FORCAM-FORCE-EDGE.pdf

Belinski, R., Peixe, A., Frederico, G., & Garza-Reyes, J. (2020). Organizational learning and industry 4.0: findings from a systematic literature review and research agenda.. Benchmarking 27(8), 2435–2457. doi:https:// doi. org/ 10. 1108/BIJ- 04- 2020- 0158

# Index of Images

Figure 1. Sub-phase A: Chuck: hard jaws.	10

Figure 2. Sub-phase B: chuck: 3 soft jaws	10

Figure 3. Grouping of surfaces associated with machining costs	14

Figure 4. Chassis view	19

Figure 5. Image for designing the manufacturing process for the robot's WS4P010 part	21

Figure 6. Analysis of the Part	21

Figure 7. Annexe: Analyse the morpholofy of the part	31

# Index of tables

Table 1. Nine steps of the manufacturing analysis	9

Table 2. Table to Assist in Choosing the Number of Machining Operations	12

Table 3. Plane cylinder and plan-plan connections	14

Table 4. Analysis of drawing specifications	22

Table 5. Decisions on the production of intrinsic surfaces	23

Table 6. Machining operation grouping table	23

Table 7. Preliminary Manufacturing Study -1  Nomenclature of phases and sub-phases	24

Table 8. Preliminary Manufacturing Study -2   Nomenclature of phases and sub-phases phase	24

Table 9. 1st Machining phase	27

Table 10. Combining surfaces	28

Table 11. Order of operations	29

Table 12. Moving into position (MiP) and holding in position (MaP)	30

Table 13. Annexe: Analyse the design drawing specifications	32

Table 14. Annexe: Making decisions on the creation of intrinsic surfaces	33

Table 15. Annexe: Define operation groupings	33

Table 16. Annexe: Preliminary Manufacturing Study -1 Nomenclature of phases and sub-phases	34

Table 17. Annexe: Preliminary Manufacturing Study -2  Nomenclature of phases and sub-phases	35

# Annexes

## Annex 1

Resource document: application rules

Table 9. 1st Machining phase

| Rule | Illustration |
| --- | --- |
| Positioning the part in the 1st machining phase must : allow good material distribution (balancing), be carried out on quality raw surfaces (avoid parting lines), facilitate effective clamping. | * Distribution of materials 1st phase |
| The surfaces machined in the first phase must contribute to the positioning of the next phase and, if possible, allow for the complete definition of the reference system. |  |
| The part, resting on the first machined surface, must be easy to clamp securely. | For the 2nd phase, the reference frame is made up of machined surfaces (plane support, centring, locating). |

Table 10. Combining surfaces

| Rule | Illustration |
| --- | --- |
| For surfaces with delicate specifications, give priority to: |  |
| - Machining these surfaces in the same phase, if they can be combined |  |
| - Machining one surface by positioning it on the other. |  |
| - The machining of these 2 surfaces concerning the same reference frame |  |
| During the machining phase, the maximum number of accessible surfaces compatible with the machine and its equipment must be machined | Machining on a horizontal machining centre |
| Fragile and expensive finishing tools should attack and, if possible, clear machined surfaces | The drawing is machined before machining. |
| The finishing tool must work alone | To finish, bore Ø 30h7 must be machined alone |

Table 11. Order of operations

| Rule | Illustration |
| --- | --- |
| The choice of the number of operations required to produce an elementary surface depends on the surface condition and the quality of the desired dimensions. The roughing operation eliminates the surface layer of metal whose physical and geometrical defects are the result of the blank manufacturing process. The semi-finishing operation ensures the regularity of the finishing chip and the geometric accuracy of position. The finishing operation must meet the required quality (dimensions, tolerances, surface finish). |  |
| Make a general rough sketch before starting to finish fragile and precise surfaces (risk of deformation, heat generation). (Drafts of A and D before finishing) |  |
| If burrs are formed, they must be rejected on a non-functional surface. | You have to put up a front before you can argue. |
| In the case of intersecting holes, the small diameter should be drilled first to avoid deflecting the tools. | Machine B before A |
| Boring and tapping operations must be preceded by a chamfering operation. |  |

Table 12. Moving into position (MiP) and holding in position (MaP)

| Rule | Illustration |
| --- | --- |
| Workpiece positioning must be isostatic. However, when machining deformable parts, additional adjustable supports can be added to prevent any deformation. |  |
| The aim should be to achieve maximum stability of the part by keeping the support points as far apart as possible. |  |
| The clamping forces must not deform the workpiece. |  |
| The supports must resist cutting forces, except in the case of ceiling mounting where the feed forces are low. | Ceiling mounting |

## Annex 2: CORRECTION

### Analysis of the part

#### Analyse the morpholofy of the part

Objective: make the most likely choice for the first phase

Locate the raw and machined surfaces using the markings defined below.

Start with the reference surfaces.

Identify the surfaces that remain rough and mark them on the Bi drawing.

Identify the machined surfaces and mark them on the drawing: plane Pi, cylinder Ci, other Xi.

C1 (x14)

B1

4

5

1

6

Directions d’usinage

3

2

X1

B2

Figure 7. Annexe: Analyse the morpholofy of the part

#### Analyse the design drawing specifications

OBJECTIVE : To identify the risks of scrap and related surfaces.

##### Complete the analysis table below

To be completed at 4.2.1 ↓

Table 13. Annexe: Analyse the design drawing specifications

| Surface mark | Reference element | Number | Specification | Tolerance zone | Link with specified reference(s) or other element(s) | Machining direction (see 2.1.1) |
| --- | --- | --- | --- | --- | --- | --- |
| B1 |  | 1 | 3 | ± 0,1 | B2 |  |
| B2 |  | 1 | 3 | ± 0,1 | B1 |  |
| C1 |  | 14 | 5 100 80 180 11 20 | ± 0,1 ± 0,1 ± 0,1 ± 0,1 ± 0,1 ± 0,1 | C1 C1 C1 X1 X1 | 1,4 |
| X1 (Contour de la pièce) |  | 1 | R1,6 R3 R240 33,4 31 | ± 0,1 ± 0,1 ± 0,1 ± 0,1 ± 0,1 |  | 1, 4 |

##### Highlight the tricky specifications.

That is to say: IT ≤ 0.05; quality ≤ 7; Ra ≤ 0.8; tolerance zone ≤ 0.05

Highlight them in the table on the previous page.

### Defining Surfaces Groupings

Some surfaces cannot be machined separately; they are grouped together by technological requirement (grooving, facing, etc.)

#### 8.2.2.1 Identify the machining directions

Complete the “Machining directions” column in table 4.1.2.

##### 8.2.2.2. Complete the surface grouping table below

#### 2.2.2.3. Making decisions on the creation of intrinsic surfaces

OBJECTIVE : to ensure surface quality.

Some surfaces may require 1/2 finishing and finishing operations, or the use of special machining processes, such as grinding.

Complete the “Comments” column of the surface grouping table

Table 14. Annexe: Making decisions on the creation of intrinsic surfaces

| Machining direction | Surface grouping | Comments |
| --- | --- | --- |
| 1 | C1, X1 |  |
| 2 |  |  |
| 3 |  |  |
| 4 | C1, X1 |  |
| 5 |  |  |
| 6 |  |  |

### Define Operation Groupings

OBJECTIVE : limit rework and handling errors

Complete the table of machining operation groupings

Table 15. Annexe: Define operation groupings

| Group of operations | Surfaces |
| --- | --- |
| Drilling, Contouring | C1, X1 |

### Order the Machining Phases

- Draw up the machining range (Fill in a range sheet for each Preliminary Manufacturing Design)

#### Propose a Preliminary Manufacturing Study

Table 16. Annexe: Preliminary Manufacturing Study -1 Nomenclature of phases and sub-phases

| Preliminary Manufacturing Study -1 Nomenclature of phases and sub-phases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N° phase | S/ Phase | Description of phases and operations | Machine | Positioning | Sketch |
| 10 |  | Raw capacity: 400x250 in plate thickness 3 |  |  |  |
| 20 |  | Pointing, C1 drilling (x14) Additional screw clamping in 4 holes 5 Contouring X1 with cutter 2 sizes | 3-axles CUCN with vertical spindle | Installed on a martyr plate Supported by flanges Supported by flanges and screws |  |

#### Propose another Preliminary Manufacturing Study

Table 17. Annexe: Preliminary Manufacturing Study -2  Nomenclature of phases and sub-phases

| Preliminary Manufacturing Study -2 Nomenclature of phases and sub-phases |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| N° phase | S/ Phase | Description of phases & operations | Machine | Positioning | Sketch |
| 10 |  | Raw capacity: 400x250 in plate thickness 3 |  |  |  |
| 20 |  | Pointing, C1 drilling to diameter 2 (x14) for wire passage | 3-axis CUCN with vertical spindle | Mounting on martyr plate Support by flanges |  |
| 30 |  | Drilling C1 to diameter 5 (x14) Contouring X1 | Wire EDM machine | Support by flanges |  |