The adapted Tknika

ACTIVITY 1

STEPS TO GET A PROPER IMAGE USING THE COGNEX 5100 CAMERA

The goal of this activity is to achieve high-quality images that allow for the correct configuration and use of a Cognex 5100 industrial camera in machine vision applications.

- Introduction

In machine vision systems, obtaining high-quality images is a must for further processing. The Cognex 5100 industrial camera is a benchmark in this field, but its proper use depends on the exact configuration of several technical factors.

- Objective

This activity is expected to develop the following competencies:

- Know the physical connection and technical configuration of an industrial camera

- Mastery of basic use of In-Sight professional software

- Identification of critical parameters to optimize image quality

- Striking a balance between clarity, focus, and exposure conditions

- METHODOLOGY

  - Physical Connection of Equipment

- A proper connection of all system components shall be ensured. The CIO-1400 power module will be used for the camera's 24V power, connecting via the black wire via the SENSOR shortcut. The configuration of the trigger system for shooting control will be carried out, in which a button will be inserted into the TRIGGER input, connecting to the Trigger (-) mass and to the 24V to Trigger (+).

- For network communication, the camera and computer will be connected via the Ethernet cable.

In order to communicate with the camera, you have to change the IP address of the computer. Computer IP address "my network sites", "view network connections", "local area connection", "properties":

  - Software and Control System

In-Sight software will be used for the management of the camera.

If the IP addresses of the camera and the computer are in the same range, we will see the two elements of the network.

Although the system offers two ways of working ("spreadsheet" and "easy builder"). It is usually worked as a spreadsheet. If the easy builder mode is activated when the program is launched, the spreadsheet can be chosen from the "view" menu.

To take an In-Sight image, we'll choose a new paper from the file menu. The system allows you to work with the camera (which is turned off in the In-Sight sensors menu) and with the computer emulator (in our case it appears in the 180-123 menu). To be able to take a photo you have to have the camera chosen.

In the menu that spreads, when we double-click on cell A0, the "shots" of the camera and its parameters will be programmed.

For the moment, as we want to achieve a unique image, it is enough to place the uprooting (shutter release) as the camera, while the automatic exposure is uprooting (shooting).
If everything has gone well, every time we activate the uprooting, we will access an image that will appear in the In-Sight software spreadsheet (Trigger).

Remember!

To see the image on the computer you have to be connected to the camera!! This is done from this icon:

On the contrary, changes to camera parameters are made from the software when we are not connected to the camera. The disconnection is done by clicking on the same icon.

Since the acquired image is not suitable for the application of visual instruments, we must improve through the parameters provided by the system, the exterior lighting and the lens entity.

  - Image quality optimization

To make the image as clean as possible, we must achieve the balance between  the three elements (exterior lighting, elements of the camera lens (aperture and aperture of the diaphragm) and camera parameters).

The process of defining optimal parameters to obtain an actionable image is convenient for the camera to be connected, as it allows you to see the image in real time as it changes with the parameters.

First of all, it is advisable to adjust the camera entity. This is done through objective roulette. The incoming light can also be controlled by opening or closing the lens diaphragm (in general, if there is a lot of light, we will close and, if there is little, we will open).

The parameters of the camera that we can control are the exposure (we usually opt for automatic exposure (the system automatically adjusts according to the light it receives)) and the lighting itself (this camera has two LED rings and each one separately can "shoot").

Once all the parameters have been adjusted, we can arrive at this image, on which we can apply the visual instruments.

  - Conclusions

- Proper use of the Cognex 5100 camera requires a number of technical capabilities; However, if set up correctly, it delivers high-quality results.

- The balance between clarity, focus and exposure parameters is essential to achieve optimal image quality.

- In-Sight software is a robust machine vision tool, but to realize its full potential requires proper basic technical configuration.