The adapted Tknika

2. ACTIVITY

SEPARATION ACTIVITY OF SUITABLE PARTS AND DECAPITANTS USING A MACHINE VISION MODEL

The objective of this activity is to develop a system for the automatic separation of suitable parts and creditors using the artificial vision model tool (pattern), based on the application of quality control.

- Introduction

Quality control in modern industry is a critical process in which machine vision systems play an important role in the automatic inspection of parts. Knowledge per model allows the system to learn the characteristics of a reference part and compare other parts.

- Objective

Through this activity, the following competencies will be developed:

- Mastery of the specialized use of the TrainPatMaxPatter professional tool

- Know the optimal settings for the FindPatMaxPatterns algorithms

- Technical tuning of critical knowledge parameters by model

- Development of the automatic quality control system

- METHODOLOGY

  - Imaging

- The application of the visual instruments will require the execution of the process carried out in activity 1.

  - Model Definition (Suitable Part)

- On the image taken,  we will apply the visual tool TrainPatMaxPattern (model-trainer). Thanks to this tool, the system will be able to remember the (appropriate) shape of the part, which it will then search for in the entire field of vision.

- We'll find the tool in the tool shovel and drag it into a cell in the spreadsheet.

- We have to choose which field the right part is in (that's what we're going to use as a reference to distinguish good and bad pieces). In this case, the pattern will be a complete piece, but a significant part of a piece can also be chosen.

- To be able to choose the space, we will click twice on the "pattern" area of the tool's property sheet.

- To program the tool, it is not necessary to set any further parameters. If we accept it, we will arrive at this screen:

- The tool is programmed in cell A2, and in cell B2, the system informs us that everything has been developed correctly.

  - Search for parts according to the model

- On the image taken, the visual tool FindPatMaxPatterns must be applied. To do this, the tool will be dragged to the cell where the calculation will be performed.

- You have to choose the search field (usually it coincides with the camera's field of view). As in the previous case, you have to enable this option by clicking on the "search region" parameter.

- To finish configuring the instrument, you need to define several sets of parameters:

  - Skipper Training Room (A2)

  - Quantity to be searched (maximum number of parts similar to the pattern the system is looking for, 8)

  - Number of acceptances (which sets the acceptable difference between the standard and the pieces to be tested, 75 has been chosen)

  - Start and end of the angle (-180 and +180 to support parts rotated according to the position of the predefined pattern)

If everything has been done correctly, the result will be as follows:

Three pieces have been found that resemble the trained pattern (suitable pieces); therefore, the system identifies the presence of 2 pieces of dismemberment. To see the result as it appears on the screen, copy cells B4-G4 and 5º, 6. and 7. They have stuck to income, changing the number of the index.

  - Conclusions

- Knowledge by model provides an effective quality control system. The combination of TrainPatMaxPattern and FindPatMaxPatterns makes it possible to distinguish the right pieces from the decorative pieces from the decorative pieces, with the appropriate parameter settings.

- In the system, three suitable and two defective parts have been identified that show the accuracy and reliability of the algorithms in this industrial application.