The adapted Tknika

5. ACTIVITY

Combined use of machine vision tools

In this activity, different artificial vision tools will be combined: first the location and orientation of a part will be detected and then the number of pins will be counted. Coordination between instruments allows for the development of a smarter system in which detection and verification are performed automatically.

- Introduction

In machine vision applications, it is often necessary to combine tools to identify the location and orientation of parts and, on the other hand, to analyze their physical characteristics. This activity will use the TrainPatMaxPattern and FindPatMaxPattern tools  to detect the location of parts, and then  compute the number of pins of an electronic repair through the ExtractBlobs tool.

- Objective

Through this activity, the following competencies will be developed:

- Training and search for a pattern with 360º angular tolerance

- Blob detection and detection via the ExtractBlobs tool

- Coordination between the different tools and establishment of the data relationship

- Automatic verification of the number of pins in a part

- METHODOLOGY

  - Imaging

As we have seen so far, the process of 1st activity must always be carried out to have an adequate image. In this case, the goal is to look for a pattern, regardless of the physical position of the object to be inspected (position other than the trained pattern and angle).

  - Pattern Training

Whatever the relative position of the part, in order to perform the image management process, we will first train the search pattern.

The image shows the  pattern chosen by the TrainPatMaxPattern function (A3 gelaxka). The objective of the practice, regardless of the position that is entered in the camera's field of view, is to look for elements in accordance with this pattern.

Using the FindPatMaxPattern tool, we will narrow down the search field for this pattern and set the search angle of the pattern (different positions of the part) appropriately. In this case, we want to do a 360º search. To do this, we indicate the beginning of the -180º angle and the end of the 180º angle.

  - Pattern tracking

Next, we will search for the pee of an electronic component. To do this,  we will use the ExtractBlobs tool  (look for the contrast of whites and grays).Once the 

ExtractBlobs tool is selected, we will click on the "region" function and delimit the area corresponding to the piens of the component.

In the image you can see that you get three different blobs that match the pins of the ingredient.

Within the properties of the ExtractBlobs function, we will select the element function and indicate the coordinates of the search pattern to follow (we will relate the ExtractBlob function  to the FindPatMaxPattern function).

If we click on the "OK" ledge, we will get this screen.

We have implemented a function in cell A16, which will return 1 if the number of pins is correct; in this case, 3 pins.

In this image you can see the result of cell A15 = 0, in the case of a 2-pin component:

  - Conclusions

- The TrainPatMaxPattern and FindPatMaxPatter instruments reliably identify the location and orientation of a part.

- The ExtractBlobs tool has a great ability to detect and analyze blobs if the parameters are configured correctly.

- The data link between the different tools (pattern position → blob search) improves the accuracy and automation of the system.

- The GetNfound feature allows you to automatically check the number of pins, making quality control processes easier.