The adapted Tknika

7. ACTIVITY

MANAGE THE VISUALIZATION PROCESS FROM A PLC THROUGH THE INTEGRATED PORT. PROFINET-IO

A Cognex camera and a Siemens PLC have been integrated into this activity via PROFINET-IO. The PLC has controlled the activation of the camera and the exchange of data. The camera has calculated the radius of the hole control panel and the number of external holes and sent the data to the PLC. The PLC, in turn, has sent the radius tolerance values to the chamber. Finally, the camera has separated the good pieces from the ones of imperossimiles in a logical condition.

- Introduction

In the automated industry, machine vision and PLC-to-PLC communication are critical for the development of intelligent processes. A Cognex camera and a Siemens PLC will be integrated into this activity via PROFINET-IO. The PLC will control the activation of the camera and the exchange of data, and the camera will measure the characteristics of the parts: the diameter of the hole control panel and the number of external holes.

- Objective

Through this activity, the following competencies will be developed:

- Control of a Cognex camera via a PLC using PROFINET-IO

- Calculation of the control panel's diameter by image and number of external holes

- Sending data from the camera to the PLC (readings) and from the PLC to the camera (tolerances)

- Automatic separation of good and decayed parts under logical conditions

- METHODOLOGY

  - Imaging

The process carried out in activity 1 must always be carried out to have an adequate image. In this case, the image should allow us to quantify the number of outer holes and calculate the diameter of the central hole.

Since the uprooting of the camera will be done via the PROFINET bus (shooting), the image uprooter must be configured:

The Arraiventador has been set to "External".

  - Camera Programming

Pattern training (TrainPatMaxpattern). To prevent the parts of the future from being in the position of the part we used to program the spreadsheet, we will train the camera to look for the part, whatever the position of the part.

The only parameter configured is the region in which the part is located.

Finding the pattern in FindPatMaxPatter. When a new part arrives, the system knows the part and sets the coordinates of its position. Subsequently, we will be able to use the coordinates in the programming of the rest of the tools.

To program the tool, the parameters defined were: zone (full zone), pattern (room A2) and start and end of the angle (-180 and +180).

Calculation of the diameter of the central hole. FINDCIRCLE tool . Thanks to this instrument, we can find a circle in a certain field and it translates its radius for us.

The following parameters have been defined for the programming of the tool:
 Element: information that sets the position of the part (C5, D5, Room E5).
Ring space: search area, two circles, between which is the search system.

Search for external holes. ExtractBlobs tool . Once the radius of the central hole has been calculated, we will program the search for 4 external holes.

The following parameters have been defined for the programming of the tool:
 Element: information that sets the position of the part (C5, D5, Room E5).
Region: it will be a square with the 4 holes that you have to look for.
Max Area Limit.: It has been adapted to the year 2000 so that the tool does not look for the central hole.

The search for external holes  is done with the GetNfound (A12) function. This function returns the number of holes (Blobs) in cell A22.

  - PLC Programming

With the Siemens software we will configure all the elements of the PLC that we will use. Care must be taken with the implementation of the IP address, which must be at the same level as the camera and the computer.

To configure the PROFINET IO network, it incorporates a "Maestro" system.

Incorporation of the Chamber into the Simatic project. As it is not a Siemens element, we must incorporate its characteristics into the HWConfig. These features are found in a GSD file that is installed on PC along with the IN-Sight Explorer software.

Once the catalog is updated, we will add it to the Cognex 5100 camera network. It is important to remember the input and output directions that the camera covers within the system, which we are going to use to communicate.

Next, we will load the configuration into the PLC, and if everything has developed correctly, the system will be ready to process the images.

  - Taking photos of the PLC

Once the entire system is configured, we can download the camera from the PLC. The first thing we have to program is the uprooting of the camera.

The A2.0 and A2.1 outputs match the enabled Camera input and trigger, respectively. According to the program made, from the E0.0 and E0.1 inputs of the PLC the uprooted chamber (disparatu) is uprooted.

  - Send the camera data to the PLC

We want to send the radius of the central hole and the number of external holes to the PLC. To do this, first of all, we must configure the format of the data to be sent in the spreadsheet.

In this case, we will send 2 pieces of data: the first, the radius of the hole (32 bittes in floating point –4 bytes) and the second is the number of outer holes (1 byte).

The send function has been programmed in room A28. When the incident occurs (collect the image), we will send the data stored in Buffer (A25).

We can check that the PLC device has correctly received the information through a table of variables.

It is important to docinize the information that comes from the camera. In our case, the first byte with information is 284, and since the first data is 4 bytes, the number of outer holes is 288 bytes.

  - Sending PLC Data to the Camera

From the PLC we want to send the upper and lower forgiveness of the radius of the central hole of the part so that the camera decides if it is a good part (the central hole inside the forgiveness and 4 external holes) or if it is bad. First, we will have to adapt the PLC program to send these two pieces of information.

The program will put the value of 90 ratings (the minimum forgiveness of the radius) in ad276 (the first output address for the data to be sent) and 91 value (maximum forgiveness) in AD280 (four bytes after the first data, since the system defines this data as real moving point numbers).

Once this is done, you need to set up the data collection from the camera spreadsheet. As always, the Buffer (FormatInputBuffer – A31 gelaxka) is defined first. In our case, the camera will receive two pieces of data (real numbers in floating point – 4 bytes).

Next, we will configure the tool that reads the data (ReadProfinetBuffer – A34 gelaxka).

The camera has correctly read the two pieces of data sent from the PLC. The lower and upper pardons are kept in cells C34 and C35.

Finally, we will have to program the spreadsheet to decide if it is a good piece visualized by the camera (the radius is inside the forgiveness and the piece has 4 holes outside) or if it is bad.

In the gela A38 we have used the AND function. This function will return a 1 if the part is correct (it meets three requirements) or a 0 if it does not meet any of the conditions.

  - Conclusions

- The Cognex Camera I/O module is useful for automating and controlling visual processes.

- ReadDiscrete and WriteDiscrete allow you to interact with external signals.

- Decisions based on image analysis are directly reflected in digital outputs.

- This activity allows artificial vision and industrial automation to work in an integrated way.